Australian Optical Society Meeting (AOS)

2-D Array Structures in Planar Waveguides Using Periodic Hypersensitisation

# J. Canning1, P. Hu1, Matteo Dainese2, Harendra Fernando2, Jayanta K. Sahu2, Lech Wosinski2

1Optical Fibre Technology Centre (OFTC), University of Sydney

Australian Photonics Cooperative Research Centre (APCRC), Australia

2 The Laboratory of Photonics and Microwave Engineering, Department of Electronics, Royal Institute of Technology, Electrum 229, S-164 40 Kista, Sweden

The fabrication of a 2-D periodic Bragg structure in a germanosilicate planar waveguide using periodic hypersensitisation and direct grating writing within a germanosilicate preform is demonstrated. This is the first demonstration of 2-step patterned photosensitivity by holographic processing of silicate glass. Coherent interference of scattered light is used to determine the presence of a weak 2-D periodic index modulation.