Interest Group:	Australian Optical Society

Title:		Developments in the Precise Determination of Im(f) for Medium-Z metals: Molybdenum.

Format:		Oral or Poster

Authors:	Martin DE JONGE, Chanh TRAN, Chris CHANTLER, Bipin DHAL, Zwi BARNEA

Affiliation:	The University of Melbourne

Postal Address:	School of Physics, The University of Melbourne, 3010.

Tel./Fax.	+61 3 83445465	/	+61 3 93494783

e-mail:	dejonge@optics.ph.unimelb.edu.au

Knowledge of the Atomic Form Factor (AFF) underpins all models of the interaction of light with atomic and molecular matter.  As such, it is necessary to obtain good experimental and theoretical data to determine these interactions for use in other quantitative investigations.  This is an ongoing concern of the International Union of Crystallography, and we have undertaken to determine sources of systematic error that have plagued earlier measurements whilst continuing to refine theoretical calculations.

We will present out latest experimental results for Molybdenum which feature an absolutely calibrated energy scale, and determinations of the effects of systematics in the attenuation measurement due to harmonic components to a level of 1 photon in 104, dark currents and detector linearities.  Sample thickness calibration has been performed by numerous independent methods including weight/area analysis, micrometry, 2-D X-ray scanning and optical interferometry.  The range of the attenuation measurement has also been extended to far exceed the Nordfors range of 2 < ln ( Io/ I ) < 4, resulting in an increase of precision and accuracy to below 0.1%.

Other features of the determination include the absolute measurement of the XAFS structure of Molybdenum in a 0.5 eV mesh which may be used to extend the understanding of the origin of the XAFS structure.

Preliminary results of an absolute measurement of the total attenuation coefficient of Molybdenum.  Note the high grid densities employed near the K-edge fine structure.