Australian Optical Society

Reflection of laser-cooled atoms from high quality, micron-period, grooved magnetic mirrors

R.J. McLEAN1, A.I. SIDOROV1, D.S. GOUGH1, F. SCHARNBERG1, B.A. SEXTON2, T.J. DAVIS2, A.M. AKULSHIN3, G.I. OPAT3 AND P. HANNAFORD1

1Centre for Atom Optics and Ultrafast Spectroscopy, School of Biophysical Sciences and Electrical Engineering, Swinburne University of Technology, PO Box 218, Hawthorn, VIC 3122

2CSIRO Manufacturing Science and Technology, Private Bag 33, Clayton South VIC 3169

3School of Physics, University of Melbourne, VIC 3010

High quality atomic mirrors and beamsplitters that permit the manipulation of slowly moving atoms are needed in atom optics.  We have an established programme of developing and testing mirrors for atom optics based on the interaction between the magnetic dipole moment of an atom and the exponentially varying magnetic field above a periodic array of magnetic elements [1].  Previously we have observed strong reflection of cold atoms from mirrors based on several different approaches [2].  For good specularity from a “hard” magnetic surface, and so that the magnetic structures can be used as diffractive beamsplitters, it is necessary that they have periodicities on a micron scale.

We have recently produced very promising high quality, micron-period mirrors based on grooved, perpendicularly magnetised Co0.8Cr0.2 structures. Structures with periods ranging from 0.7 to 4 m have been fabricated using electron beam lithography and replication techniques, and characterised with magnetic force microscopy.  Laser-cooled Cs and Rb atoms have been reflected from structures of area 1010 mm2.  Time-of-flight observations of optically-pumped atoms show that the mirrors are highly reflecting, and analysis of the spatial profiles of the falling and bouncing atom clouds indicate that the angular spread resulting from the reflection process is consistent with good specularity. This type of magnetic structure appears to be the most promising to date for producing high-quality magnetic mirrors with micron periodicity.  Their quality could be improved further by using magneto-optical materials, which offer superior magnetic characteristics.

References

- G.I. Opat, S.J. Wark, A. Cimmino,  Appl. Phys. B 54, 369 (1992).

- D.C. Lau, R.J. McLean, A.I. Sidorov, D.S. Gough, J. Koperski, W.J. Rowlands, B.A. Sexton, G.I. Opat and P. Hannaford,  J. Opt B: Quantum Semiclass. Opt. 1, 371 (1999)