Investigation of effect of surface film in the white-light interference microscopes

M. ROY†,  P. MOORE†,  C.J.R. SHEPPARD†* and P. HARIHARAN†

†University of Sydney, NSW 2006 Australia

*Australian Key Centre for Microscopy and Microanalysis,

University of Sydney, NSW 2006, Australia

mroy@physics.usyd.edu.au

Interference microscopes using monochromatic light run into phase ambiguities if the test surface has steps whose height is greater than a wavelength.  This problem can be avoided by using white light and scanning the test surface along the height axis (Davidson et al. 1987).  The variations in intensity at each point in the image can then be processed to find the peak of the fringe-visibility curve.  The location of this  peak along the scanning z-axis then corresponds to the height of the surface at this point.

A major application area of white light interference microscopes (WLIM) is the investigation of integrated structures in the semiconductor device and photonics industry.  However, such surfaces are sometimes covered with thin films of transparent materials such as oxides and sulphides, which can significantly change the spectral reflectance of the surface [2, 3].  We show how these changes can introduce ambiguities with WLIM based on Michelson interferometer configuration.  The spectrum of the reflected field from the surface of a mirror having various thickness of dielectric film and the deviations of the position of maximum visibility including multiple reflections inside the film will be discussed.  Figure 1. shows changes in the visibility curve for SiO2 film on Si substrate with increasing optical thickness of the surface film.  Here we emphasize the need for caution in interpreting the results when there is possibility of the formation of thin films.

Figure1.

# References

iquid Crystal (FLC) devices.  [1]  M. Davidson, K. Kaufman, I. Mazor and F. Cohen, 1987, SPIE, 775,  233-247.

[2]  P. Hariharan and M. Roy, 1996,  J. Mod. Opt., 43, 1797

[3]  C. J. R. Sheppard, T. J. Connoly, J. Lee and C.J. Cogswell, 1994, App. Opt., 33, 631-640